Приложение 10. Список комплектующих для подбора

Имеющиеся компоненты:

1. Рама Mark4, 5 дюймов

Name: REA-Mark4-5inch

Weight: 102g

Motor to moto: 225mm

Size: 145*172mm

Weapon Plate: 5mm

Top plate: 2.5mm

Bottom plate: 2.5mm

Side plate: 2.5mm

Camera mount size: 19.3mm

2. Полётный контроллер

Product Name: F4 V3 PLUS flight controller

Firmware: OMNIBUSF4SD

Size: 36x36mm

Mounting holes: 30.5x30.5mm

STM32 F405 MCU

Dshot Support

Drag OSD customized via Betaflight

MPU: MPU6500 / MPU6000

SBUS/PPM and Spketrum DSMX ports

MicroSD blackbox card

Baro (BMP280)

5v3a 9V3A BEC

Built-in video filters (only 5V to VTX and camera)

3. Двигатели

Недостающие компоненты:

1. Регуляторы оборотов

Вариант 1

Model: Skywalker-40A

Continuous current: 40A

Current Break (>10s): 55A

BEC mode: Linear

BEC output: 5V2A

BEC output power: 5 servos (2S Lipo) / 4 servos (3S Lipo)

Battery: 2-3S (Lipo) / 5-9 cells (NiMH)

Net weight: 35g

Dimensions (L x WxH): 68x25x8mm

Вариант 2

1: Initial voltage: 2-6s lipo 
2: no BEC 
3: constant current: 30A, pulse current: 35A to 10s 
4: Size: 35*17mm. 
5: Weight: 9g (including power and signal wire) 
6: Support mute mode, Oneshot125, Oneshot42, Multshot and DSHOT 
7: Main IC: SILABS EFM8BB21F16, MCU run 48MHz 
8: MOS: low and high side fets are Nfets 
9 A: The switching speed is very fast, it has a special driver chip 
10: BLHeli S bootloader and firmware

Вариант 3

Specification:

Item type: 4 in 1 45A ESC
Weight: Approx.46g/1.6oz
Material: electronic components
Color: black
ESC size: about 44x43mm/1.73x1.69in
Fixed hole: Approx.20x20mm/0. 8 x 0.8in, Approx.30.5 x 30.5mm/1.2 x 1.2in
Fixed iris: M4
Operating voltage: 2-6S
Maximum continuous operating current: 45A
Maximum instantaneous current: 50A
Support: DSHOT150, 300, 600, PWM , Oneshot125, Oneshot42 and Multshot
Software: blhelisuit
Firmware version: BL16.7
Application: suitable for RC drones and FPV traversing vehicle, etc.

2. Пропеллеры

| Вариант 1 | Вариант 2 | Вариант 3 |
| --- | --- | --- |

3. АКБ

Вариант 1

Вариант 2

Вариант 3

Пример электронной формы подбора комплектующих